Software Compatibility

The CD contains the latest Remote Control Software RCS 2.0.4 and the firmware for the D-01, KM D, TLM103 D, KMR 81 D, KMS104 D and KMS105 D digital microphones as well as the firmware for the 2 channel Digital Microphone Interfaces DMI-2 and DMI-2 portable as well as the 8 channel Digital Microphone Interface DMI-8 and the EtherSound enabled Multi Channel Audio Interface MCA-ES.

Please note: In order to use the new features you need to update all Solution-D components in the sequence RCS, DMI, microphones.

Software compatibility to RCS 2.0.4 is given using the following firmware version:

| Device | Software Version | File Name |
| --- | --- | --- |
| D-01 (hardware version 12.02) | 5.14 | D-01_HW12_02_SW05_14.mfw |
| D-01 (hardware version 13.03) | 6.05 | D-01_HW13_03_SW06_05.mfw |
| KM D | 2.13 | KM D_HW01_02-04_SW02_13.mfw |
| TLM103 D | 1.07 | TLM103 D_HW01_00-01_SW01_07.mfw |
| KMR 81 D | 1.03 | KMR 81 D_HW01_02-04_SW01_03.mfw |
| KMS104 D | 1.01 | KMS104 D_HW01_02-04_SW01_01.mfw |
| KMS105 D | 1.01 | KMS105 D_HW01_02-04_SW01_01.mfw |
| DMI -2 (hardware version 003) | 5.08 | DMI2_HW003_SW05_08.dfw |
| DMI -2 portable | 1.06 | DMI2_PORTABLE_HW001_SW01_06.dfw |
| DMI-8 (hardware version 001) | 1.38 | DMI8_HW001_SW01_38.dfw |
| DMI-8 (hardware version 002) | 1.44 | DMI8_HW002_SW01_44.dfw |
| MCA-ES | 1.00 | MCA_ES_HW001_SW01_00.dfw |

Update Procedure

### 1. Update RCS to revision 2.0.4

- exit the RCS if running

- launch ‘Setup.exe’ (Windows) or ‘Install RCS’ (Mac) from the CD and follow the instructions of the installer

### 2. Update the DMI-2/8

- launch RCS 2.0.4

- click ‘Options’  ‘DMI’ in the menu

- if the ‘Software revision’ is older than shown in the table above then update the DMI by choosing the button ‘SW Update’ and selecting the corresponding file from the CD

### 3. Update the microphone(s)

- click the button ‘System’ and the System window will appear

- click the button ‘SW Update’ in the System window and the ‘SW Update’ window will appear

- if the ‘Version’ of the applied boot partition is older than shown in the table above then choose the appropriate microphone firmware file from CD

- select the target boot partition in the ‘Save To’ column

- click the button ‘Start Update’. The microphone firmware download needs up to 35 min

- when the download is completed close the ‘SW Update’ window

- after that it takes about one minute until the microphone is ready for use again. This is indicated by the Data Valid LED on the DMI-2 and the corresponding display in the RCS

Georg Neumann GmbH, February 26, 2019